# Computer Networks Lab

## Lab 5: Switching and ARP (MAC Address Learning)

### 1. Objective

• Understand switching in networks.
• Learn how data travels between PCs via switches.
• Understand ARP (Address Resolution Protocol).
• Observe MAC address table behavior.
• Learn to view and clear MAC address table.

### 2. Basic Theory

A switch is a Layer 2 device that forwards frames using MAC addresses. If the destination MAC is unknown, the switch floods the frame.

ARP (Address Resolution Protocol) maps IP addresses to MAC addresses before communication.

### 3. MAC Address Table

The switch stores MAC addresses in a MAC (CAM) table. It updates automatically when devices communicate.

### 4. Required Software

• Cisco Packet Tracer

### 5. Procedure

Part A – Network Setup
• Create a network with 2 PCs and 1 switch.
• Optionally add another switch.
• Connect using straight-through cables.
• Assign IP addresses in same network.

Part B – Check MAC Table
• Open switch CLI.
• Run: show mac address-table

Part C – Communication
• Ping from one PC to another.

Part D – Verify Table
• Run: show mac address-table
• Observe learned MAC addresses.

Part E – Clear Table
• Run: clear mac address-table dynamic
• Verify using show command.

### 6. Expected Result

Students should understand MAC learning, ARP process, and switching behavior.

### 7. Lab Task

• Create network with PCs and switch.
• Assign IP addresses.
• Check MAC table before ping.
• Ping between PCs.
• Check MAC table after ping.
• Clear MAC table.
• Verify clearing.

### 8. Submission Instructions

• Perform task.
• Take screenshots.
• Paste in Word file.
• Write short explanation.
• Submit to instructor.